# Nema 17 Stepper Motor 17HS3401

17HS3401 4-lead Nema 17 Stepper Motor 42 motor 42BYGH 1.3A CE ROSH ISO CNC Laser and 3D printer

Product Description:
Model: 17HS3401
Phase: 2
Step Angle (deg): 1.8
Motor Length (mm): 34
Rated Current (A): 1.3
Phase Resistance (Ohm): 2.4
Phase Inductance (mH): 2.8
Holding Torque (N.cm): 28

Detent Torque (N.cm): 1.6
Rotor Inertia (g.cm2): 34
Lead Wire (No): 4
Motor Weight (g): 220

Motor Frame Size: 43*34mm
Shaft: “D” 21mm length 5mm diameter
Outlet way: Plug line

Step angle accuracy: + – 5% (full step, not load)
Resistance accuracy: + – 10%
Inductance accuracy: + – 20%
Temperature rise: 80deg Max (rated current, 2 phase on)
Ambient temperature: 20deg ~ +50deg
Insulation resistance: 100MΩ Min, 500VDC
Insultion Strength: 500VAC for one minute

Pinouts:
Red: A+
Green: A-
Yellow: B+
Blue: B-